xiii

THE 8086 INSTRUCTION SET

Before I go through the instructions I need to say something

about the structure of the instructions. For the majority of

instructions, two bits in the first instruction byte and the

whole second byte determine whether it is a byte or word

instruction and whether it is:

1. register, register

2. register, memory

3. memory, register

These refer to the 8 arithmetic registers, not the segment

registers. "Memory" is any allowable addressing mode. Therefore,

it is simpler to abbreviate this as:

reg/mem, reg/mem

This will always mean:

1. either a byte or word operation is allowed

2. any of the above 3 possibilities is allowed

unless specifically stated otherwise. Another possibility is the

operations with constants:

add  ax, 7

xor  sign_flag, 80h

These follow the same form. There are two possibilities:

1. register, constant

2. memory, constant

where this may be a byte or a word, "register" is any arithmetic

register, and "memory" is any allowable addressing mode. These

will be abbreviated:

reg/mem, constant

If we are taliking about a segment register, they will be

abbreviated:

segreg

If anything does not follow this pattern, it will be explicitly

stated.

DESTINATION,SOURCE

The standard way of writing instructions is with the destination

on the left and the source on the right:

______________________

The PC Assembler Tutor - Copyright (C) 1989 Chuck Nelson

The PC Assembler Tutor                                        xiv

______________________

add  destination, source

The register or memory location on the left ALWAYS gets the

result of the operation. The thing on the right is the second

operand (if any).

ADDRESSING MODES

These are the natural (default) segments of all addressing modes:

(1) DS

variable + (constant)

[bx] + (constant)

[si] + (constant)

[di] + (constant)

[bx+si] + (constant)

[bx+di] + (constant)

(2) SS

[bp] + (constant)

[bp+si] + (constant)

[bp+di] + (constant)

Where the constant is optional. Segment overrides may be used.

The segment overrides are:

SEGMENT OVERRIDE         MACHINE CODE (hex)

CS:                      2E

DS:                      3E

ES:                      26

SS:                      36

These default segments apply to all instructions except the

string instructions, which will be explained individually.

THE INSTRUCTION SET

AAA  (ascii adjust for addition) adjusts AL, assuming that it

contains the result of a legitimate unpacked BCD addition. If the

lower half-byte has generated a result 10 or over, it subtracts

10, carries 1 to AH, and sets the carry flag. If the result is 9

or less, it clears CF. In either case it zeroes the upper

half-byte of AL.

AAD (ascii adjust for division) PREPARES AL and AH for division.

It assumes that AH contains the 10's digit and AL contains the

1's digit of a two byte unpacked BCD number. It multiplies AH by

10 and adds it to AL, thus making a single integer between 0 and

99. It zeroes AH in preparation for unsigned division.

Appendix II - The 8086 Instruction Set                         xv

______________________________________

AAM (ascii adjust for multiplication) adjusts AL, assuming that

it contains the result of a legitimate BCD multiplication. It

divides the result by 10, putting the quotient in AH and the

remainder in AL. If AL contains 73 before AAM, then afterwards AH

will contain 7 and AL will contain 3

AAS  (ascii adjust for subtraction) adjusts AL, assuming that it

contains the result of a legitimate unpacked BCD subtraction. If

the lower half-byte has generated a result -1 or less, it borrows

1 from AH, adds 10 to AL, and sets the carry flag. If the result

is 0 or more, it clears CF. In either case it zeroes the upper

half-byte of AL.

ADC (add with carry) adds two integers, either signed or

unsigned, and adds in the carry from the previous arithmetic

operation. It is used for multiple word (byte) addition.

adc  reg/mem, reg/mem

adc  reg/mem, constant

ADD adds two integers, either signed or unsigned.

add  reg/mem, reg/mem

add  reg/mem, constant

AND ands two bytes or words. A bit remains set only if both its

respective source and destination bits were set.

and  reg/mem, reg/mem

and  reg/mem, constant

CALL calls a procedure. The two forms we have used are NEAR and

FAR calls.

call set_regs

call FAR PTR calculate_array

There are two other forms where the addresses of the subprograms

are in memory and changable. You should only use these forms if

you are writing an operating system or a compiler, since they are

an invitation to disaster.

CBW sign extends the signed byte in AL to AH:AL in preparation

for signed division. It is used alone without any register

specification.

cbw

The PC Assembler Tutor                                        xvi

______________________

CLC clears the carry flag (CF = 0).

CLD clears the direction flag (increments string pointers).

CLI clears the interrupt flag (disables interrupts). All maskable

interrupts must wait till the flag is set again before they will

be processed.

CMC changes the value of the carry flag. If CF =1, then CF = 0;

if CF = 0, then CF = 1.

CMP compares two values. It is the same as SUB except it does not

alter the "destination" variable. Its job is to set the flags as

if a subtraction had been performed, in preparation for a

conditional jump instruction.

cmp  reg/mem, reg/mem

cmp  reg/mem, constant

CMPS compares the byte (or word) at DS:[si] with the one at

ES:[di], (calculating [si] - [di]) and sets the flags

accordingly. It increments (or decrements) SI and DI depending on

the setting of DF, the direction flag (by 1 for bytes and by 2

for words). You may use CS:[si], SS:[si] or ES:[si], but you MAY

NOT OVERRIDE ES:[di]. Notice that SI and DI are reversed from

their position in the other string instructions.

cmpsb

cmpsw

cmps BYTE PTR SS:[si], ES:[di]     ;or CS, DS, ES:[si]

cmps WORD PTR SS:[si], ES:[di]     ;or CS, DS, ES:[si]

CWD sign extends the signed integer in AX to DX:AX in preparation

for signed word division. No register specification is used.

cwd

DAA (decimal adjust for addition) adjusts AL, assuming that it

contains the result of a legitimate packed BCD addition. It

treats AL as two independent half-bytes. If the result of the

lower half-byte is 10 or over, it subtracts 10 from the lower

half-byte and adds the carry to the upper half byte. It then

looks at the upper half byte. If its result is 10 or over, DAA

subtracts 10 from the upper half byte and sets the carry flag.

Appendix II - The 8086 Instruction Set                       xvii

______________________________________

Otherwise the carry flag is cleared.

DAS (decimal adjust for subtraction) adjusts AL, assuming that it

contains the result of a legitimate packed BCD subtraction.It

treats AL as two independent half-bytes. If the result of the

lower half-byte is -1 or less, it adds 10 to the lower half-byte

and borrows 1 from the upper half byte. It then looks at the

upper half byte. If its result is -1 or less, DAA adds 10 to the

upper half byte and sets the carry flag to indicate a borrow.

Otherwise the carry flag is cleared.

DEC decrements the byte or word by 1. It does not alter CF.

dec  reg/mem

DIV performs unsigned division. If byte division, the unsigned

double byte must be in AH:AL. Afterwards, AL has the quotient and

AH has the remainder. If word division, the unsigned word must be

in DX:AX. Afterwards, AX has the quotient, and DX the remainder.

Division by a constant is not allowed. The DIV instruction does

not mention DX:AX or AH:AL. They are understood.

div  reg/mem

ESC (escape) signals to the 8086 that the bytes starting with the

ESC byte are a coprocessor instruction. The 8086 will calculate a

memory address (if appropriate) and give it to the coprocessor.

The 8086 will then go on to the next instruction.

HLT (halt) halts the machine. It can be restarted with a reset, a

non-maskable interrupt, or (if IEF is set) with a maskable

interrupt.

IDIV performs signed integer division. For byte division, the

value in AL must be sign extended to AH (with CBW), giving the

double signed byte AH:AL, after division, AL contains the

quotient and AH contains the remainder. For word division, the

value in AX must be sign extended to DX (with CWD), giving the

double signed word DX:AX, after division, AX contains the

quotient and DX contains the remainder. Division by a constant is

not allowed. IDIV cannot perform multiple word division. To do

that, you need to make the numbers positive and use DIV,

adjusting the signs afterwards. IDIV does not mention the AL or

AX register, it is understood.

The PC Assembler Tutor                                      xviii

______________________

idiv reg/mem

IMUL performs signed integer multiplication. The multiplicand

must be in AX (or AL for bytes). After the multiplication, the

result is in DX:AX for words and AH:AL for bytes. If DX (or AH

for bytes) contains significant information, then CF is set (=1).

If all the information is in AX (or AL for bytes) then CF = 0.

Multiplication by a constant is not allowed. The IMUL instruction

does not mention AX (or AL); it is understood.

imul mem/reg

IN allows you to move data from a port address to AX (for a word)

or AL (for a byte). There are two forms:

IN   al, port_number

IN   al, dx

Where 'port_number' is a CONSTANT that is hard coded into the

machine instruction and is from 0 - 255. The port address in DX

may be from 0-65535. It must be the DX register.

INC increments a register or a variable by 1. It does not effect

CF.

inc  reg/mem

INT (interrupt) sends the program to a subprogram whose address

is located in the interrupt vector table in low memory. For any

interrupt number I, the 8086 goes to segment 0000, offset

(I X 4). It loads IP with the first two bytes, then loads CS from

the next two bytes. The next instruction executed will be at the

new CS:IP. Before loading the new CS and IP, it pushes (1) the

flags, (2) the old CS and (3) the old IP in that order.

int  3

int  21h

INTO checks OF. If OF = 1, INTO generates interrupt 4. Otherwise

it does nothing. It is used for error handling of signed numbers.

into

IRET (interrupt return) is a special return instruction for

interrupt routines. It pops (1) the old IP, (2) the old CS, and

(3) the old flags in that order.

Appendix II - The 8086 Instruction Set                        xix

______________________________________

iret

JUMPS ---------------------------------------------------------

There are two kinds of jumps. JMP is unconditional and can jump

anywhere in the segment. All other jumps are conditional and are

limited jumps from -128 to +127 bytes from the END of the jump

instruction.{1}

THESE ARE THE JUMP INSTRUCTIONS FOR SIGNED NUMBERS

jg        ; jump if greater

jnle      ; jump if not (less or equal)

jl        ; jump if less

jnge      ; jump if not (greater or equal)

je        ; jump if equal

jz        ; jump if zero

jge       ; jump if greater or equal

jnl       ; jump if not less

jle       ; jump if less or equal

jng       ; jump if not greater

jne       ; jump if not equal

jnz       ; jump if not zero

THESE ARE THE JUMP INSTRUCTIONS FOR UNSIGNED NUMBERS

ja        ; jump if above

jnbe      ; jump if not (below or equal)

jb        ; jump if below

jnae      ; jump if not (above or equal)

je        ; jump if equal

jz        ; jump if zero

jae       ; jump if above or equal

jnb       ; jump if not below

jbe       ; jump if below or equal

jna       ; jump if not above

____________________

1 There is also a long jump which can jump anywhere in memory

(from 00000 to FFFFF). Except under special circumstances, using

this is truly lousy programming practice. The long jump is for

the Olympics, not for quality programming.

The PC Assembler Tutor                                         xx

______________________

jne       ; jump if not equal

jnz       ; jump if not zero

THESE JUMPS CHECK A SINGLE FLAG

These come in opposite pairs

jc        ; jump if the carry flag is set

jnc       ; jump if the carry flag is not set

jo        ; jump if the overflow flag is set

jno       ; jump if the overflow flag is not set

jp or jpe ; jump if parity flag is set (parity is even)

jnp or jpo  ;jump if parity flag is not set (parity is odd)

js        ; jump if the sign flag is set (negative )

jns       ; jump if the sign flag is not set (positive or 0)

THIS CHECKS THE CX REGISTER

jcxz      ; jump if cx is zero

This is used before entry to a loop to make sure the loop counter

is not set to 0.

INDIRECT JUMPS are jumps where the information for the jump is

stored in memory (or in a register for an in-segment jump). These

forms are dangerous and should be used only when writing things

like multi-tasking operating systems. Have you written one

lately?

END OF JUMPS  - - - - - - - - - - - - - - - - - - - - - - -

LAHF (load AH from flags) loads the lower half of the flags

register into AH.

LDS (load DS) loads the first two bytes of the memory address

into the named arithmetic register and the next two bytes into

DS. The register may be any arithmetic register, but this is

designed for the 4 addressing registers: BX, SI, DI and BP.

lds  reg, memory_address

LEA calculates an offset address and puts it in the named

register.

Appendix II - The 8086 Instruction Set                        xxi

______________________________________

lea  ax, [bp+si]+145

LES (load ES) loads the first two bytes into the named arithmetic

register and the next two bytes into ES. The register may be any

arithmetic register, but this is designed for the 4 addressing

registers: BX, SI, DI and BP.

les  reg, memory_address

LOCK is for use if there is more than one 8086 operating in a

computer. LOCK allows one 8086 to be the only one which can read

from or write to memory during the following instruction.

LODS moves a byte or word from DS:[si] to AL or AX, and

increments (or decrements) SI depending on the setting of DF, the

direction flag (by 1 for bytes and by 2 for words). You may use

CS:[si], SS:[si] or ES:[si].

lodsb

lodsw

lods BYTE PTR SS:[si]         ; or CS, DS, ES:[si]

lods WORD PTR SS:[si]         ; or CS, DS, ES:[si]

LOOP/LOOPE/LOOPNE are conditional jumps (-128 to + 127 bytes).

They will jump back to the start of the loop (which is identified

by a label), under the following conditions:

loop      decrement cx ; jump back if cx is not zero

loope     decrement cx ; jump back if cx not zero AND zf = 1

loopz     decrement cx ; jump back if cx not zero AND zf = 1

loopne    decrement cx ; jump back if cx not zero AND zf = 0

loopnz    decrement cx ; jump back if cx not zero AND zf = 0

Here, 'e' stands for equal, 'z' is zero and 'n' is not.

loop some_label

MOV is the general instruction for moving bytes or words on the

8086. The forms are:

reg/mem, reg/mem

reg/mem, constant

segreg, reg/mem

reg/mem, segreg

Notice that you may not (1) move a constant to a segment

register, (2) move a segment register to another segment

The PC Assembler Tutor                                       xxii

______________________

register, or (3) move from memory to memory.

MOVS moves a byte (or a word) from DS:[si] to ES:[di], and

increments (or decrements) SI and DI depending on the setting of

DF, the direction flag (by 1 for bytes and by 2 for words). You

may use CS:[si], SS:[si] or ES:[si], but you MAY NOT OVERRIDE

ES:[di].

movsb

movsw

movs BYTE PTR ES:[di], SS:[si]     ;or CS, DS, ES:[si]

movs WORD PTR ES:[di], SS:[si]     ;or CS, DS, ES:[si]

MUL performs unsigned integer multiplication. The multiplicand

must be in AX (or AL for bytes). After the multiplication, the

result is in DX:AX (or AH:AX for bytes). If DX (or AH for bytes)

contains significant information, then CF = 1. If all the

significant information is in AX (or AL for bytes) then CF = 0.

Multiplication by a constant is not allowed. The MUL instruction

does not mention AX (or AL). It is understood.

mul  reg/mem

NEG performs '0 - number' on a number and sets the flags

accordingly.

neg  reg/mem

NOP (no operation) does absolutely nothing. It can fill up space

which was previously occupied by a different instruction.

NOT performs bitwise logical NOT on a word or a byte.

not  reg/mem

OR performs bitwise logical OR on a byte or word

or   reg/mem, reg/mem

or   reg/mem, constant

OUT moves a byte (or word) from AL (or AX) to the named port.

There are two forms:

Appendix II - The 8086 Instruction Set                      xxiii

______________________________________

out  port_number, ax

out  dx, ax

where 'port_number' is a CONSTANT which is coded into the machine

code. The constant may be from 0-255. DX (and it must be DX) may

hold a port number from 0-65535.

POP pops a WORD off the stack. Technically, POP takes the word at

SS:SP and puts it into the named register or memory location,

then ADDS 2 to SP.

pop  mem/reg

pop  segreg

POPF pops a WORD off the top of the stack into the flags

register. Its technical operation is the same as for POP.

PUSH pushes a WORD on the stack from the named register or memory

location. Technically, PUSH subtracts 2 from SP, then moves the

word to SS:SP.

push reg/mem

push segreg

PUSHF pushes the flags register on the stack. Its technical

operation is the same as for PUSH.

RCR (rotate through carry right) and RCL (rotate through carry

left) rotate the bits of a register or of memory data right and

left respectively. The bit which is shoved off the register (or

data) is placed in CF and CF is placed on the other side of the

register (or data). There are two fixed forms. (1) rotate 1 bit

and (2) rotate by the number in CL.

rcr  reg/mem, 1

rcl  reg/mem, cl

REP. The string instructions may be prefixed by REP/REPE/REPNE

which will repeat the instructions according to the following

conditions:

rep       decrement cx ; repeat if cx is not zero

repe      decrement cx ; repeat if cx not zero AND zf = 1

repz      decrement cx ; repeat if cx not zero AND zf = 1

repne     decrement cx ; repeat if cx not zero AND zf = 0

The PC Assembler Tutor                                       xxiv

______________________

repnz     decrement cx ; repeat if cx not zero AND zf = 0

Here, 'e' stands for equal, 'z' is zero and 'n' is not. These

repeat instructions should NEVER be used with a segment override,

since the 8086 will forget the override if a hardware interrupt

occurs in the middle of the REP loop.

rep    movsb

repe   scasb

repne  cmpsw

RET returns from the subroutine to the calling program. The

return will be either NEAR or FAR depending on the type of

procedure it is in. It may take the parameters off the stack (for

Pascal) or leave them on the stack (for C).

ret (6)        ; Pascal - take 6 bytes off the stack

ret            ; C - do nothing

ROR (rotate right) and ROL (rotate left) rotate the bits of a

register or memory data right and left respectively. The bit

which is shoved off one end is moved to the other end. CF

indicates whether the last bit moved from one end to the other

was a 1 or a 0. There are two fixed forms. (1) rotate 1 bit and

(2) rotate by the number in CL.

ror  reg/mem, 1

rol  reg/mem, cl

SAHF (store AH into flags) puts AH into the low byte of the flags

register.

SAL (shift arithmetic left) and SHL (shift logical left) are

exactly the same instruction. They move bits left. 0s are placed

in the low bit. Bits are shoved off the register or memory data

on the left side, and CF indicates whether the last bit shoved

off was a 1 or a 0. It is used for multiplying an unsigned number

by powers of 2. There are two fixed forms. (1) shift 1 bit and

(2) shift by the number in CL.

shl  reg/mem, 1

sal  reg/mem, cl

SAR (shift arithmetic right) shifts bits right. The high (sign)

bit stays the same throughout the operation. Bits are shoved off

the register or memory location on the right side. CF indicates

whether the last bit shoved off was a 1 or a 0. It is used (with

Appendix II - The 8086 Instruction Set                        xxv

______________________________________

difficulty) for dividing a signed number by powers of 2. There

are two fixed forms. (1) shift 1 bit and (2) shift by the number

in CL.

sar  reg/mem, 1

sar  reg/mem, cl

SBB (subtract with borrow) subtracts one integer from another and

then subtracts 1 more if CF is set. It works with both signed and

unsigned numbers and is used for multiple word arithmetic.

sbb  reg/mem, reg/mem

sbb  reg/mem, constant

SCAS compares AL (or AX) to the byte (or word) pointed to by

ES:[di], and increments (or decrements) DI depending on the

setting of DF, the direction flag (by 1 for bytes and by 2 for

words).  NO OVERRIDES ARE ALLOWED.

scasb

scasw

scas BYTE PTR ES:[di]    ; no override allowed

scas WORD PTR ES:[di]    ; no override allowed

SHR (shift logical right) does the same thing as SHL but in the

opposite direction. Bits are shifted right. 0s are placed in the

high bit. Bits are shoved off the register or memory location on

the right side and CF indicates whether the last bit shoved off

was a 0 or a 1. It is used for dividing an unsigned number by

powers of 2. There are two fixed forms. (1) shift 1 bit and (2)

shift by the number in CL.

shr  reg/mem, 1

shr  reg/mem, cl

STC sets the carry flag (CF = 1).

STD sets the direction flag (DF = 1, which decrements).

STI sets the interrupt flag (IEF = 1; interrupts enabled).

STOS (store to string) moves a byte (or a word) from AL (or AX)

to ES:[di], and increments (or decrements) DI depending on the

setting of DF, the direction flag (by 1 for bytes and by 2 for

words). NO SEGMENT OVERRIDES ARE ALLOWED.

stosb

The PC Assembler Tutor                                       xxvi

______________________

stosw

stos BYTE PTR ES:[di]    ; no override allowed

stos WORD PTR ES:[di]    ; no override allowed

SUB subtracts one integer from another. It works for both signed

and unsigned numbers.

sub  reg/mem, reg/mem

sub  reg/mem, constant

TEST performs logical AND, but does not alter the value of the

"destination" variable. Its purpose is to set the flags for

conditional jumps.

test reg/mem, reg/mem

test reg/mem, constant

WAIT forces the 8086 to wait until the coprocessor is finished

with its instruction before the 8086 can go on to the next

instruction.

XCHG exchanges the values in two registers or memory locations.

xchg reg/mem, reg/mem

XLAT. If BX contains the address of a 256 byte translation table,

then XLAT goes to (BX + AL), takes the byte there and puts it

into AL. DS is the normal segment, but segment overrides are

allowed.

XOR performs logical exclusive OR on two numbers.

xor  reg/mem, reg/mem

xor  reg/mem, constant